The SCSILINK package v0.50

--------------------------

DISTRIBUTION

============

This software package is copyright Roger Burrows, but may be freely

distributed; you are encouraged to give it to others, but at no cost.

DISCLAIMER

==========

I am advised to include a disclaimer because there are unreasonable

people out there.

"I make no warranty with respect to this documentation, or the program

it describes, and disclaim any implied/explicit suggestions of usefulness

for any particular purpose.  Use this program only if you are willing to

assume all risks, and damages, if any, arising as a result, even if it is

caused by negligence or other fault."

*** WARNING ***

===============

This is the first public beta of the MintNet driver for the Daynaport

SCSI/Link device.  As usual with beta test software, there may be bugs

in it that could cause data loss.  Therefore, before running this

software for the first time, we advise you to take a backup of your

system.  In addition, we recommend taking backups on a regular basis

(but you were doing that already, right?).

PROGRAM DOCUMENTATION

=====================

SCSILINK.XIF

============

This is a driver for MintNet that supports the Daynaport SCSI/Link (a

SCSI-to-Ethernet device).

Hardware installation

---------------------

1. Make sure your system is powered off.

2. Connect the Daynaport SCSI/Link into the SCSI chain of your TT or

Falcon.  If it is physically the last device in the chain, make sure

that termination is on (the 1-0 switch is set to 1), otherwise, make

sure that it's off (the switch is set to 0).  Set the SCSI id (the

rotary switch) to a value that is different from any other device in

the chain, and different from the id of the system itself, which is

normally 7.

3. Connect the Ethernet side of the device into your Ethernet (if you

are using 10baseT, you will need a hub).

4. Power on all the devices in the SCSI chain, before powering on your

system.

Software installation

---------------------

1. Read carefully the "WARNING" above!

2. Make sure that you have a SCSIDRV driver on your system; recent

versions of HDDRIVER provide this function.

3. Make sure that arbitration is enabled for the SCSIDRV driver, and

that the id of the SCSI/Link device is being controlled by SCSIDRV.

4. Copy SCSILINK.XIF to the same folder as your other XIF files; with

a current MiNT distribution, that's MINT/1-16-CUR.

5. Update the various configuration files:

/etc/sysconfig/config.if

/etc/sysconfig/domainname

/etc/sysconfig/hostname

/etc/sysconfig/defaultroute

/etc/resolv.conf

Marc-Anton Kehr's e-nick program is a convenient way to do this.

Many thanks to Marc-Anton for this and his wonderful EasyMiNT

distribution!

5. Reboot MiNT.  You should see two 'sign-on' messages from the driver

during initialisation; they should look something like this:

DaynaPORT SCSI/Link driver v0.50 initialising ...

DaynaPORT SCSI/Link driver v0.50 (en0) (xx:xx:xx:xx:xx:xx) on bus id 1/4

SLINKCTL.TTP

------------

SLINKCTL is a command-line program for controlling SCSILINK.XIF.    It

retrieves statistics, clears statistics, and/or resets the device.  The

statistics are intended to help in problem determination, so will not be

very meaningful to the end user; however, if you have a problem, we may

ask you to run it and send the results to Anodyne Software.  SLINKCTL

has the following syntax:

slinkctl [-c] [-r] [-tN] <interface> [filename]

By default (i.e. with no parameters) it reports internal statistics

including the trace for the interface named <interface>.  If one or more

of the options is set, the appropriate actions are carried out but no

report is created.

Available options:

-c	clears the statistics counters

-r	resets the interface (useful if it seems to be hung)

-tN	sets the number of entries in the trace table to N; N must

be between 0 & 10000 inclusive.  NOTE: the trace table was

created to investigate a particular problem during driver

development, and is unlikely to be of general use.  It's

recommended that you leave the number of entries at zero

(the default).

Output is to stdout, unless a filename is specified, in which case

all output will be written to it instead.

Examples:

slinkctl.ttp en0 stats.txt

writes the statistics for port 'en0' to file stats.txt

slinkctl.ttp -c en0

clears the statistics for port 'en0'

slinkctl.ttp -r en1

resets the SCSI/Link device at port 'en1'

PERFORMANCE

===========

Performance was measured by using FTP to transfer a large (~10MB) file

between a TT and a Windoze box.  The following rates were obtained:

a) FTP server on Windoze, client on TT

PC->TT     130 KB/sec

TT->PC     153 KB/sec

b) FTP server on TT, client on Windoze

PC->TT     107 KB/sec

TT->PC     142 KB/sec

Transfers *to* the TT are expected to be slower than transfers from it,

since the TT has to poll the SCSI/Link hardware for an incoming packet,

rather than handle an interrupt.  Polling is currently triggered via

the VBL interrupt; triggering from the system timer interrupt would

improve performance at a cost of increased overhead.

REVISION HISTORY

================

Version

-------

0.50        First public beta release

NOTES

=====

1. Testing to date has been on a TT running TOS 3.06, with the SCSI/Link

connected to the SCSI port; however, it should also work on the ACSI

port (see below), and on other TOS-compatible systems including the

Falcon.  Please let me know if you have had success or problems under

other configurations.

2. If you wish to use the ACSI port of an ST, Mega, or TT, you will

need (at least) a host adapter that supports SCSI arbitration, i.e.

a Link96/Link97.  You will also need a device on that port which

provides TERMPWR to power the Link9x (the SCSI/Link does not).

3. SCSILINK.XIF is written to support multiple devices (ethernet ports),

but that feature has not yet been tested.  If you connect two devices

to the same Atari system and it works, please let me know!

Roger Burrows

Anodyne Software

11/March/2007