CheckRTC v3.0 (13 August 2005)

------------------------------

This program is Freeware by Roger Burrows, Anodyne Software.

Distribute freely, but please keep this document with the program!

Introduction

------------

Many, if not all, TT030s seem to have a real time clock that loses

time almost as badly as PC systems :-).  This program was written

to quantify how bad the problem was.  This latest version also

checks the real time clock on Falcon 030s and Mega ST(e)s.

Function

--------

This program compares the elapsed time as measured by the 200Hz system

tick counter, and as measured by the Real Time Clock.  The elapsed time

starts when the program is invoked, and is compared each time you press

a key, at which time a message is issued noting any discrepancies.

Ideally, the times ought to agree within 1 second/day; in order to

verify to this degree of accuracy, you will need to let CHECKRTC run

for at least 8 minutes.

Experience has shown that the clock in Mega ST/Mega STe/Falcon030

systems is typically accurate within 1-2 seconds/day; however, many

(but not all) TTs have clocks that are 10-20 seconds slow per day.

This program is for the Mega ST, Mega STe, TT030 and Falcon030 only.